- Throttle Mode			F3A w/GOV

- Throttle Channel		Not Used (Read the throttle data from the bus when the PWM throttle channel is not connected;)

- Spool Up Factor		3 (default)

- Spool Down Factor		7 (default)

- Before the first time you use GOV mode, reset the transmitter throttle range and perform throttle channel calibration (only once)

- Caution: Please perform throttle channel calibration in F3A/IMAC mode instead of GOV mode (which can be dangerous).

- The following parameters are for the CRS3000 MKII/MKIII & CRS3500 MKIII(AE)

- Motor Timing			16 degrees (default for 100XHV)

- ESC Frequency			24KHz and up (default)

- Motor Poles			14

- Reduction Gear Ratio	4.77

- Min Prop RPM			800

- Max Prop RPM		4000

- Gov P-Gain			6 of 20

- Gov I-Gain			13 of 20

- Below: See GOV in action with Jeti's Log feature, which helps to optimize Gain P&I values